# Protocol: Bioreactor Modeling in gPROMS

Prepared 2026-10-24

## Overview

This protocol builds a fed-batch bioreactor model in gPROMS, simulates it, fits its kinetic parameters to lab data, and optimizes the feed. gPROMS is an equation-oriented modeling platform from Siemens (formerly Process Systems Enterprise); this protocol uses the general-purpose gPROMS language, as in gPROMS ModelBuilder or gPROMS Process.

Example system: an aerobic microbial fermentation with biomass (X), substrate (S) and product (P), Monod growth, Luedeking–Piret product formation, and a glucose feed that raises the volume.

Workflow:

- Create a project and define variable types.

- Write the MODEL: parameters, variables and equations.

- Write the PROCESS: parameter values, inputs, initial conditions and schedule.

- Run the dynamic simulation and check the results.

- Estimate kinetic parameters from experimental data.

- Optimize the feed profile to maximize product.

Menu names and some keywords differ slightly between gPROMS versions. Check them against the documentation for your installed version.

## Requirements

- Software: gPROMS ModelBuilder or gPROMS Process with a valid licence. Parameter estimation and optimization may need extra licence features.

- Background: mass balances, Monod kinetics, ordinary differential equations.

- Data for Step 6: time-course measurements of X, S and P (g/L) from at least 2 batch or fed-batch runs, with feed rate, feed concentration and starting volume recorded.

- Starting guesses for parameters:

| Parameter | Symbol | Typical starting value | Unit |
| --- | --- | --- | --- |
| Maximum growth rate | μmax | 0.4 | 1/h |
| Half-saturation constant | Ks | 0.5 | g/L |
| Biomass yield on substrate | Yxs | 0.5 | g/g |
| Growth-associated product coefficient | α | 0.2 | g/g |
| Non-growth product coefficient | β | 0.01 | g/(g·h) |
| Maintenance coefficient | ms | 0.02 | g/(g·h) |

These are generic textbook values for a bacterial fermentation. Replace them with values for your organism.

## Steps 1–2: Project and variable types

### Step 1: Create the project

- Open gPROMS and choose File → New to create a project, for example FedBatch_Bioreactor.gPJ.

- In the project tree you will add entities under these folders: Variable Types, Models, Processes, and later Experiments and Parameter Estimations.

- Save often. Each entity is checked for syntax when you save or build.

### Step 2: Define variable types

A variable type sets the default value, lower and upper bounds and unit for a class of variables. Bounds stop the solver from wandering into impossible values such as negative concentrations.

Right-click Variable Types → New Entity, then add one row per type:

| Name | Default | Lower bound | Upper bound | Unit |
| --- | --- | --- | --- | --- |
| Concentration | 1 | 0 | 500 | g/L |
| Volume | 1 | 0 | 100 | L |
| Flowrate | 0 | 0 | 10 | L/h |
| SpecificRate | 0.1 | -10 | 10 | 1/h |
| Mass | 1 | 0 | 1E4 | g |

## Step 3: Write the bioreactor MODEL

The model is 4 mass balances plus 1 kinetic expression. F is the feed rate and Sf the feed substrate concentration.

dVdt=F

dXdt=μX-FVX

dSdt=FVSf-S-μYxs+msX

dPdt=αμ+βX-FVP

μ=μmaxSKs+S

Right-click Models → New Entity, name it FedBatchReactor, and enter:

# Fed-batch bioreactor: Monod growth, Luedeking-Piret product
PARAMETER
  mu_max  AS REAL     # maximum specific growth rate, 1/h
  Ks      AS REAL     # half-saturation constant, g/L
  Yxs     AS REAL     # biomass yield on substrate, g/g
  alpha   AS REAL     # growth-associated product term, g/g
  beta    AS REAL     # non-growth product term, g/(g h)
  ms      AS REAL     # maintenance coefficient, g/(g h)
  Sf      AS REAL     # feed substrate concentration, g/L

VARIABLE
  X   AS Concentration   # biomass
  S   AS Concentration   # substrate
  P   AS Concentration   # product
  V   AS Volume          # broth volume
  F   AS Flowrate        # feed rate (input)
  mu  AS SpecificRate    # specific growth rate
  D   AS SpecificRate    # dilution rate F/V
  Ptot AS Mass           # total product in reactor

EQUATION
  # Kinetics
  mu = mu_max * S / (Ks + S) ;
  D  = F / V ;

  # Mass balances
  $V = F ;
  $X = mu * X - D * X ;
  $S = D * (Sf - S) - (mu / Yxs + ms) * X ;
  $P = (alpha * mu + beta) * X - D * P ;

  # Output of interest
  Ptot = P * V ;

Degrees of freedom: 8 variables, 7 equations. F is the one input you must set in the PROCESS. gPROMS reports the count when you build; it must be zero before a simulation will run.

## Steps 4–5: PROCESS entity and simulation

### Step 4: Write the PROCESS

The PROCESS sets parameter values, fixes the input, gives initial conditions and runs a schedule. Here the reactor runs as a batch for 8 h, then feeds at 0.05 L/h for 16 h.

Right-click Processes → New Entity, name it Sim_FedBatch, and enter:

UNIT
  R101 AS FedBatchReactor

SET
  WITHIN R101 DO
    mu_max := 0.4 ;
    Ks     := 0.5 ;
    Yxs    := 0.5 ;
    alpha  := 0.2 ;
    beta   := 0.01 ;
    ms     := 0.02 ;
    Sf     := 400 ;     # g/L glucose in feed
  END

ASSIGN
  WITHIN R101 DO
    F := 0 ;            # batch phase, no feed
  END

INITIAL
  WITHIN R101 DO
    X = 0.1 ;           # inoculum, g/L
    S = 20 ;            # initial glucose, g/L
    P = 0 ;
    V = 2 ;             # starting volume, L
  END

SOLUTIONPARAMETERS
  ReportingInterval := 0.25 ;   # h

SCHEDULE
  SEQUENCE
    CONTINUE FOR 8                 # batch phase, h
    RESET R101.F := 0.05 ; END     # start feeding, L/h
    CONTINUE FOR 16                # fed-batch phase, h
  END

### Step 5: Run and check

- Select Sim_FedBatch and click Simulate (the run button).

- Read the execution output. Fix any errors before looking at results:

  - Syntax errors point to the entity and line.

  - A degrees-of-freedom error means an input is missing from ASSIGN or assigned twice.

  - A failed initialization usually means bad initial values or bounds.

- Open the trajectories in the results viewer (or export to Excel or CSV) and plot X, S, P and V against time.

- Sanity checks:

  - S never goes below 0 and X rises while S falls.

  - V rises by exactly 0.05 × 16 = 0.8 L, from 2.0 to 2.8 L.

  - A carbon or mass balance closes within your expected error.

- Save the results set with a clear name, for example Base_case_F0.05.

## Step 6: Parameter estimation

Fit μmax, Ks, Yxs, α, β and ms to your measured X, S and P. gPROMS uses maximum-likelihood estimation, so it also reports confidence intervals and correlations.

- Prepare the PROCESS for estimation. Copy Sim_FedBatch to PE_Base. Keep the parameters to be estimated in SET with their starting guesses; gPROMS overrides them during fitting.

- Create one Performed Experiment per lab run (right-click Experiments → New Entity). For each run enter:

  - Initial conditions: X, S, P and V at time 0.

  - Controls: the feed rate F over time, as piecewise-constant steps matching what you actually fed.

  - Measurements: time, variable (R101.X, R101.S, R101.P) and measured value. Paste these from Excel.

- Create the Parameter Estimation entity (right-click Parameter Estimations → New Entity) and set:

  - The PROCESS to use: PE_Base.

  - Parameters to estimate, each with starting guess, lower and upper bound (for example μmax 0.4, bounds 0.05–1.5 1/h).

  - Experiments to include: all Performed Experiments.

  - Measurement variance model for each sensor, for example constant variance (absolute error) for S and relative variance for X.

- Run the estimation and read the report.

- Judge the fit:

  - Plot model curves against data for every experiment.

  - Check each parameter’s 95 % confidence interval and t-value. A t-value below the reference t means the data cannot pin that parameter down.

  - Look at the correlation matrix. Values above about 0.95 (μmax with Ks is common) mean the pair is not separately identifiable.

  - Check the lack-of-fit (weighted residual) test against its χ² reference value.

- If the fit is poor: fix poorly identifiable parameters at literature values, add a richer experiment (for example a pulse of substrate), or extend the model (for example substrate inhibition).

- Validate on a run that was not used for fitting.

- Copy the final estimates back into the SET section of the simulation PROCESS.

## Step 7: Dynamic optimization of the feed profile

Find the feed rate over time that gives the most product at the end of the run, within the reactor’s volume limit.

- Copy the fitted PROCESS to Opt_Base.

- Create an Optimisation entity (right-click Optimisations → New Entity) linked to Opt_Base.

- Objective: maximize R101.Ptot at the final time.

- Control variable: R101.F, piecewise-constant over, for example, 8 intervals of 3 h. Bounds 0–0.15 L/h.

- Time horizon: fixed at 24 h, or let gPROMS treat the final time as a decision variable with bounds such as 16–36 h.

- Constraints:

  - End-point: R101.V ≤ 4 L (working volume).

  - Path: R101.S ≤ 5 g/L, to avoid overflow metabolism or inhibition.

- Run the optimization. Start from the base-case feed as the initial guess.

- Check the result: plot the optimal F profile and the resulting X, S, P and V. Confirm all constraints are met and the profile is physically sensible (no rapid on–off switching you cannot run).

- Test robustness: re-run with μmax and Yxs at the edges of their confidence intervals and check that product stays acceptable.

- Write the optimal profile into a new simulation PROCESS (Sim_Optimal) using RESET steps, so the recipe can be re-run and shared.

## Troubleshooting and good practice

| Problem | Likely cause | Fix |
| --- | --- | --- |
| “Degrees of freedom not zero” | Input missing from ASSIGN, or a variable assigned and also given by an equation | Count variables and equations; assign exactly the free inputs (here F) |
| Initialization fails | Initial values far from the solution, or a division by zero (V = 0) | Set realistic INITIAL values and variable-type defaults; keep V > 0 |
| Variable hits a bound | Bounds too tight, or a real model error (S going negative) | Check the balance signs first, then widen bounds if justified |
| Integration stalls when S runs out | Monod term becomes stiff near S = 0 | Use a small positive lower bound, or a smooth switch, and tighten solver tolerances |
| Parameter estimates at their bounds | Poor identifiability or poor starting guesses | Fix some parameters, change guesses, add informative experiments |
| Optimal feed is bang-bang and unrunnable | Too many control intervals or no penalty on changes | Use fewer intervals or add a rate-of-change constraint |

Good practice:

- Keep units consistent (g, L, h) and write them in comments next to every parameter and variable.

- Build in stages: batch model first, then add feeding, then product, checking results each time.

- Keep separate PROCESS entities for simulation, estimation, and optimization so a change to one does not break the others.

- Version the project file and record parameter sets with the date and data used.

- Document the final model, data sources, and assumptions so others can reproduce the results.

The gPROMS code here is written from general knowledge of the language; verify syntax and menu names against your version’s documentation.